Note: This document contains FAR Part 108 including Amendment 108-19 as published in the Federal Register on Oct. 10, 2000.

PART 108	AIRPLANE OPERATOR SECURITY SPECIAL FEDERAL AVIATION REGULATIONS

SFAR 58 -Advanced Qualification Program

108.1  Applicability.

108.3  Definitions.

108.4  Falsification.

108.5  Security program: Adoption and implementation.

108.7  Security program: Form, content, and availability.

108.9  Screening of passengers and property.

108.10  Prevention and management of hijackings and sabotage attempts.

108.11  Carriage of weapons.

108.13  Security of airplanes and facilities.

108.14  Transportation of Federal Air Marshals.

108.15  Law enforcement officers.

108.17  Use of X-ray systems.

108.18  Security Directives and Information Circulars.

108.19  Security threats and procedures.

108.20  Use of Explosives Detection Systems.

108.21  Carriage of passengers under the control of armed law enforcement escorts.

108.23  Training.

108.25  Approval of security programs and amendments.

108.27  Evidence of compliance.

108.29  Standards for security oversight.

108.31  Employment standards for screening personnel.

108.33  Access investigation.

Authority: 49 U.S.C. 106(g), 5103, 40113, 40119, 44701-44702, 44705, 44901-

44905, 44907, 44913-44914, 44932, 44935-44936, 46105.

Source: Docket No. 108, 46 FR 3786, Jan. 15, 1981, unless otherwise noted.

SFAR No. 58

Special Federal Aviation Regulation No. 58--Advanced Qualification Program

Section

1. Purpose and eligibility.

2. Definitions.

3. Required Curriculums.

4. Indoctrination Curriculums.

5. Qualification Curriculums.

6. Continuing Qualification Curriculums.

7. Other Requirements.

8. Certification.

9. Training Devices and Simulators.

10. Approval of Advanced Qualification Program.

11. Approval of Training, Qualification, or Evaluation by a Person Who Provides Training by Arrangement.

12. Recordkeeping requirements.

13. Expiration.

Contrary provisions of parts 61, 63, 65, 121, and 135 of the Federal Aviation Regulations notwithstanding--

1.	Purpose and Eligibility.

(a)	This Special Federal Aviation Regulation provides for approval of an alternate method (known as “Advanced Qualification Program” or “AQP”) for qualifying, training, certifying, and otherwise ensuring competency of crewmembers, aircraft dispatchers, other operations personnel, instructors, and evaluators who are required to be trained or qualified under parts 121 and 135 of the FAR or under this SFAR.

(b)	A certificate holder is eligible under this Special Federal Aviation Regulation if the certificate holder is required to have an approved training program under Sec. 121.401 or Sec. 135.341 of the FAR, or elects to have an approved training program under Sec. 135.341.

(c)	A certificate holder obtains approval of each proposed curriculum under this AQP as specified in section 10 of this SFAR.

(d)	A curriculum approved under the AQP may include elements of present Part 121 and Part 135 training programs. Each curriculum must specify the make, model, and series aircraft (or variant) and each crewmember position or other positions to be covered by that curriculum. Positions to be covered by the AQP must include all flight crewmember positions, instructors, and evaluators and may include other positions, such as flight attendants, aircraft dispatchers, and other operations personnel.

(e)	Each certificate holder that obtains approval of an AQP under this SFAR shall comply with all of the requirements of that program.

2.	Definitions. As used in this SFAR:

Curriculum means a portion of an Advanced Qualification Program that covers one of three program areas: (1) indoctrination, (2) qualification, or (3) continuing qualification. A qualification or continuing qualification curriculum addresses the required training and qualification activities for a specific make, model, and series aircraft (or variant) and for a specific duty position.

Evaluator means a person who has satisfactorily completed training and evaluation that qualifies that person to evaluate the performance of crewmembers, instructors, other evaluators, aircraft dispatchers, and other operations personnel.

Facility means the physical environment required for training and qualification (e.g., buildings, classrooms).

Training center means an organization certificated under part 142 of this chapter or an organization approved by the Administrator to operate under the terms of this SFAR to provide training as described in section 1(b) of SFAR 58.

Variant means a specifically configured aircraft for which the FAA has identified training and qualification requirements that are significantly different from those applicable to other aircraft of the same make, modeI, and series.

3.	Required Curriculums. Each AQP must have separate curriculums for indoctrination, qualification, and continuing qualification as specified in sections 4, 5, and 6 of this SFAR.

4.	Indoctrination Curriculums. Each indoctrination curriculum must include the following:

(a)	For newly hired persons being trained under an AQP: Company policies and operating practices and general operational knowledge.

(b)	For newly hired flight crewmembers and aircraft dispatchers: General aeronautical knowledge.

(c)	For instructors: The fundamental principles of the teaching and learning process; methods and theories of instruction; and the knowledge necessary to use aircraft, flight training devices, flight simulators, and other training equipment in advanced qualification curriculums.

(d)	For evaluators: Evaluation requirements specified in each approved curriculum; methods of evaluating crewmembers and aircraft dispatchers and other operations personnel; and policies and practices used to conduct the kinds of evaluations particular to an advanced qualification curriculum (e.g., proficiency and online).

5.	Qualification Curriculums. Each qualification curriculum must include the following:

(a)	The certificate holder’s planned hours of training, evaluation, and supervised operating experience.

(b)	A list of and text describing the training, qualification, and certification activities, as applicable for specific positions subject to the AQP, as follows:

(1)	Crewmembers, aircraft dispatchers, and other operations personnel.

Training, evaluation, and certification activities which are aircraft- and equipment-specific to qualify a person for a particular duty position on, or duties related to the operation of a specific make, model, and series aircraft (or variant); a list of and text describing the knowledge requirements, subject materials, job skills, and each maneuver and procedure to be trained and evaluated; the practical test requirements in addition to or in place of the requirements of parts 61, 63, and 65; and a list of and text describing supervised operating experience.

(2)	Instructors. Training and evaluation to qualify a person to impart instruction on how to operate, or on how to ensure the safe operation of a particular make, model, and series aircraft (or variant).

(3)	Evaluators. Training, evaluation, and certification activities that are aircraft and equipment specific to qualify a person to evaluate the performance of persons who operate or who ensure the safe operation of, a particular make, model, and series aircraft (or variant).

6.	Continuing Qualification Curriculums. Continuing qualification curriculums must comply with the following requirements:

(a)	General. A continuing qualification curriculum must be based on—

(1)	A continuing qualification cycle that ensures that during each cycle each person qualified under an AQP, including instructors and evaluators, will receive a balanced mix of training and evaluation on all events and subjects necessary to ensure that each person maintains the minimum proficiency level of knowledge, skills, and attitudes required for original qualification; and

(2)	If applicable, flight crewmember or aircraft dispatcher recency of experience requirements.

(b)	Continuing Qualification Cycle Content. Each continuing qualification cycle must include at least the following:

(1)	Evaluation period. An evaluation period during which each person qualified under an AQP must receive at least one training session and a proficiency evaluation at a training facility. The number and frequency of training sessions must be approved by the Administrator. A training session, including any proficiency evaluation completed at that session, that occurs any time during the two calendar months before the last date for completion of an evaluation period can be considered by the certificate holder to be completed in the last calendar month.

(2)	Training. Continuing qualification must include training in all events and major subjects required for original qualification, as follows:

(i)	For pilots in command, seconds in command, flight engineers, and instructors and evaluators: Ground training including a general review of knowledge and skills covered in qualification training, updated information on newly developed procedures, and safety information.

(ii)	For crewmembers, aircraft dispatchers, instructors, evaluators, and other operation personnel who conduct their duties in flight: Proficiency training in an aircraft, flight training device, or flight simulator on normal, abnormal, and emergency flight procedures and maneuvers.

(iii)	For instructors and evaluators who are limited to conducting their duties in flight simulators and flight training devices: Proficiency training in a flight training device and/or flight simulator regarding operation of this training equipment and in operational flight procedures and maneuvers (normal, abnormal, and emergency).

(3)	Evaluations. Continuing qualification must include evaluation in all events and major subjects required for original qualification, and online evaluations for pilots in command and other eligible flight crewmembers. Each person qualified under an AQP must successfully complete a proficiency evaluation and, if applicable, an online evaluation during each evaluation period. An individual’s proficiency evaluation may be accomplished over several training sessions if a certificate holder provides more than one training session in an evaluation period. The following evaluation requirements apply:

(i)	Proficiency evaluations as follows:

(A)	For pilots in command, seconds in command, and flight engineers: A proficiency evaluation, portions of which may be conducted in an aircraft, flight simulator, or flight training device as approved in the certificate holder’s curriculum which must be completed during each evaluation period.

(B)	For any other persons covered by an AQP a means to evaluate their proficiency in the performance of their duties in their assigned tasks in an operational setting.

(ii)	Online evaluations as follows:

(A)	For pilots in command: An online evaluation conducted in an aircraft during actual flight operations under part 121 or part 135 or during operationally (line) oriented flights, such as ferry flights or proving flights. An online evaluation in an aircraft must be completed in the calendar month that includes the midpoint of the evaluation period. An online evaluation that is satisfactorily completed in the calendar month before or the calendar month after the calendar month in which it becomes due is considered to have been completed during the calendar month it became due.  However, in no case is an online evaluation under this paragraph required more often than once during an evaluation period.

- During the online evaluations required under paragraph (b)(3)(ii)(A) of this section, each person performing duties as a pilot in command, second in command, or flight engineer for that flight, must be individually evaluated to determine whether he or she--

(1)	Remains adequately trained and currently proficient with respect to the particular aircraft, crew position, and type of operation in which he or she serves; and

(2) Has sufficient knowledge and skills to operate effectively as part of a crew.

(4)	Recency of experience. For pilots in command and seconds in command, and, if the certificate holder elects, flight engineers and aircraft dispatchers, approved recency of experience requirements.

(c)	Duration periods. Initially the continuing qualification cycle approved for an AQP may not exceed 26 calendar months and the evaluation period may not exceed 13 calendar months. Thereafter, upon demonstration by a certificate holder that an extension is warranted, the Administrator may approve extensions of the continuing qualification cycle and the evaluation period in increments not exceeding 3 calendar months. However, a continuing qualification cycle may not exceed 39 calendar months and an evaluation period may not exceed 26 calendar months.

(d)	Requalification. Each continuing qualification curriculum must include a curriculum segment that covers the requirements for requalifying a crewmember, aircraft dispatcher, or other operations personnel who has not maintained continuing qualification.

7.	Other Requirements. In addition to the requirements of sections 4, 5, and 6, each AQP qualification and continuing qualification curriculum must include the following requirements:

(a)	Approved Cockpit Resource Management (CRM) Training applicable to each position for which training is provided under an AQP.

(b)	Approved training on and evaluation of skills and proficiency of each person being trained under an AQP to use their cockpit resource management skills and their technical (piloting or other) skills in an actual or simulated operations scenario. For flight crewmembers this training and evaluation must be conducted in an approved flight training device or flight simulator.

(c)	Data collection procedures that will ensure that the certificate holder provides information from its crewmembers, instructors, and evaluators that will enable the FAA to determine whether the training and evaluations are working to accomplish the overall objectives of the curriculum.

8.	Certification. A person enrolled in an AQP is eligible to receive a commercial or airline transport pilot, flight engineer, or aircraft dispatcher certificate or appropriate rating based on the successful completion of training and evaluation events accomplished under that program if the following requirements are met:

(a)	Training and evaluation of required knowledge and skills under the AQP must meet minimum certification and rating criteria established by the Administrator in parts 61, 63, or 65. The Administrator may accept substitutes for the practical test requirements of parts 61, 63, or 65, as applicable.

(b)	The applicant satisfactorily completes the appropriate qualification curriculum.

(c)	The applicant shows competence in required technical knowledge and skills (e.g., piloting) and cockpit resource management knowledge and skills in scenarios that test both types of knowledge and skills together.

(d)	The applicant is otherwise eligible under the applicable requirements of part 61, 63, or 65.

9.	Training Devices and Simulators.

- (a)	Qualification and approval of flight training devices and flight simulators.

(1)	Any training device or simulator that will be used in an AQP for one of the following purposes must be evaluated by the Administrator for assignment of a flight training device or flight simulator qualification level:

(i)	Required evaluation of individual or crew proficiency.

(ii)	Training activities that determine if an individual or crew is ready for a proficiency evaluation.

(iii)	Activities used to meet recency of experience requirements.

(iv)	Line Operational Simulations (LOS).

(2)	To be eligible to request evaluation for a qualification level of a flight training device or flight simulator an applicant must—

(i)	Hold an operating certificate; or

(ii)	Be a training center that has applied for authorization to the Administrator or has been authorized by the Administrator to conduct training or qualification under an AQP.

(3)	Each flight training device or flight simulator to be used by a certificate holder or training center for any of the purposes set forth in paragraph (a)(1) of this section must—

(i)	Be, or have been, evaluated against a set of criteria established by the Administrator for a particular qualification level of simulation;

(ii)	Be approved for its intended use in a specified AQP; and

(iii)	Be part of a flight simulator or flight training device continuing qualification program approved by the Administrator.

(b)	Approval of other Training Equipment.

(1)	Any training device that is intended to be used in an AQP for purposes other than those set forth in paragraph (a)(1) of this section must be approved by the Administrator for its intended use.

(2)	An applicant for approval of a training device under this paragraph must identify the device by its nomenclature and describe its intended use.

(3)	Each training device approved for use in an AQP must be part of a continuing program to provide for its serviceability and fitness to perform its intended function as approved by the Administrator.

10.	Approval of Advanced Qualification Program.

(a)	Approval Process. Each applicant for approval of an AQP curriculum under this SFAR shall apply for approval of that curriculum. Application for approval is made to the certificate holder’s FAA Flight Standards District Office.

(b)	Approval Criteria. An application for approval of an AQP curriculum will be approved if the program meets the following requirements:

(1)	It must be submitted in a form and manner acceptable to the Administrator.

(2)	It must meet all of the requirements of this SFAR.

(3)	It must indicate specifically the requirements of parts 61, 63, 65, 121 or 135, as applicable, that would be replaced by an AQP curriculum. If a requirement of parts 61, 63, 65, 121, or 135 is replaced by an AQP curriculum, the certificate holder must show how the AQP curriculum provides an equivalent level of safety for each requirement that is replaced. Each applicable requirement of parts 61, 63, 65, 121 or 135 that is not specifically addressed in an AQP curriculum continues to apply to the certificate holder.

(c)	Application and Transition. Each certificate holder that applies for one or more advanced qualification curriculums or for a revision to a previously approved curriculum must comply with Sec. 121.405 or Sec. 135.325, as applicable, and must include as part of its application a proposed transition plan (containing a calendar of events) for moving from its present approved training to the advanced qualification training.

(d)	Advanced Qualification Program Revisions or Rescissions of Approval. If after a certificate holder begins operations under an AQP, the Administrator finds that the certificate holder is not meeting the provisions of its approved AQP, the Administrator may require the certificate holder to make revisions in accordance with Sec. 121.405 or Sec. 135.325, as applicable, or to submit and obtain approval for a plan (containing a schedule of events) that the certificate holder must comply with and use to transition to an approved Part 121 or Part 135 training program, as appropriate.

11.	Approval of Training, Qualification, or Evaluation by a Person who Provides Training by Arrangement.

(a)	A certificate holder under part 121 or part 135 may arrange to have AQP required training, qualification, or evaluation functions performed by another person (a “training center”) if the following requirements are met:

(1)	The training center’s training and qualification curriculums, curriculum segments, or portions of curriculum segments must be provisionally approved by the Administrator. A training center may apply for provisional approval independently or in conjunction with a certificate holder’s application for AQP approval. Application for provisional approval must be made to the FAA’s Flight Standards District Office that has responsibility for the training center.

(2)	The specific use of provisionally approved curriculums, curriculum segments, or portions of curriculum segments in a certificate holder’s AQP must be approved by the Administrator as set forth in section 10 of this SFAR.

(b)	An applicant for provisional approval of a curriculum, curriculum segment, or portion of a curriculum segment under this paragraph must show that the following requirements are met:

(1)	The applicant must have a curriculum for the qualification and continuing qualification of each instructor or evaluator employed by the applicant.

(2)	The applicant’s facilities must be found by the Administrator to be adequate for any planned training, qualification, or evaluation for a part 121 or part 135 certificate holder.

(3)	Except for indoctrination curriculums, the curriculum, curriculum segment, or portion of a curriculum segment must identify the specific make, model, and series aircraft (or variant) and crewmember or other positions for which it is designed.

(c)	A certificate holder who wants approval to use a training center’s provisionally approved curriculum, curriculum segment, or portion of a curriculum segment in its AQP, must show that the following requirements are met:

(1)	Each instructor or evaluator used by the training center must meet all of the qualification and continuing qualification requirements that apply to employees of the certificate holder that has arranged for the training, including knowledge of the certificate holder’s operations.

(2)	Each provisionally approved curriculum, curriculum segment, or portion of a curriculum segment must be approved by the Administrator for use in the certificate holder’s AQP. The Administrator will either provide approval or require modifications to ensure that each curriculum, curriculum segment, or portion of a curriculum segment is applicable to the certificate holder’s AQP.

(d)	Approval for the training, qualification, or evaluation by a person who provides training by arrangement authorized by this section expires on August 3, 1998 unless that person meets the eligibility requirements specified under Sec. 121.402 or Sec. 135.324 of this chapter. After August 2, 1998 approval for the training, qualification, or evaluation, by a person who provides training by arrangement authorized by this section, shall be granted only to persons who meet the eligibility requirements specified under Sec. 121.402 or Sec. 135.234 of this chapter.

12.	Recordkeeping Requirements. Each certificate holder and each training center holding AQP provisional approval shall show that it will establish and maintain records in sufficient detail to establish the training, qualification, and certification of each person qualified under an AQP in accordance with the training, qualification, and certification requirements of this SFAR.

13.	Expiration. This Special Federal Aviation Regulation terminates on October 2, 2005, unless sooner terminated.

[Amdt. 108-18, 65 FR 60334, Oct. 10, 2000]

AIRPLANE OPERATOR SECURITY

Sec. 108.1  Applicability.

# (a)	This part prescribes aviation security rules governing—

(1)	The operations of holders of FAA air carrier operating certificates or operating certificates engaging in scheduled passenger operations or public charter passenger operations;

(2)	Each person aboard an airplane operated by a certificate holder described in paragraph (a)(1) of this section;

(3)	Each person on an airport at which the operations described in paragraph (a)(1) of this section are conducted;

(4)	Each certificate holder who receives a Security Directive or Information Circular and each person who receives information from a Security Directive or an Information Circular issued by the Director of Civil Aviation

Security; and

(5)	Each person who files an application or makes entries into any record or report that is kept, made or used to show compliance under this part, or to exercise any privileges under this part.

# (b)	This part does not apply to helicopter or to all-cargo operations.

[Doc. No. 24883, Amdt. 108-4, 51 FR 1352, Jan. 10, 1986, 
as amended bu Amdt. 108-6, 54 FR 28984, July 10, 1989; 
Amdt. 107-9, 61 FR 64244, Dec. 3, 1996]

Sec. 108.3  Definitions.

The following are definitions of terms used in this part: (a) “Certificate holder” means a person holding an FAA operating certificate when that person engages in scheduled passenger or public charter passenger operations or both.

(b)	“Passenger seating configuration” means the total number of seats for which the aircraft is type certificated that can be made available for passenger use aboard a flight and includes that seat in certain airplanes which may be used by a representative of the Administrator to conduct flight checks but is available for revenue purposes on other occasions.

(c)	‘Private charter” means any charter for which the charterer engages the total capacity of an airplane for the carriage of: (1) Passengers in civil or military air movements conducted under contract with the Government of the United States of the Government of a foreign country; or (2) Passengers invited by the charterer, the cost of which is borne entirely by the charterer and not directly or indirectly by the individual passengers.

(d)	“Public charter” means any charter that is not a “private charter.”

(e)	“Scheduled passenger operations” means holding out to the public of air transportation service for passengers from identified air terminals at a set time announced by timetable or schedule published in a newspaper, magazine, or other advertising medium.

(f)	“Sterile area” means an area to which access is controlled by the inspection of persons and property in accordance with an approved security program or a security program used in accordance with Sec. 129.25.

Sec. 108.4  Falsification.

No person may make, or cause to be made, any of the following:

(a)	Any fraudulent or intentionally false statement in any application for any security program, access medium, or identification medium, or any amendment thereto, under this part.

(b)	Any fraudulent or intentionally false entry in any record or report that is kept, made, or used to show compliance with this part, or to exercise any privileges under this part.

(c)	Any reproduction or alteration, for fraudulent purpose, of any report, record, security program, access medium, or identification medium issued under this part.

[Amdt. 107-9, 61 FR 64244, Dec. 3, 1996]

Sec. 108.5  Security program: Adoption and implementation.

(a)	Each certificate holder shall adopt and carry out a security program that meets the requirements of Sec. 108.7 for each of the following scheduled or public charter passenger operations:

(1) Each operation with an airplane having a passenger seating configuration of more than 60 seats.

(2) Each operation that provides deplaned passengers access, that is not otherwise controlled by a certificate holder using an approved security program or a foreign air carrier using a security program required by Sec. 129.25, to a sterile area.

(3)	Each operation with an airplane having a passenger seating configuration of more than 30 but less than 61 seats; except that those parts of the program effecting compliance with the requirements listed in Sec. 108.7(b) (1), (2), and (4) need only be implemented when the Director of Civil Aviation Security or a designate of the Director notifies the certificate holder in writing that a security threat exists with respect to the operation.

(b)	Each certificate holder that has obtained FAA approval for a security program for operations not listed in paragraph (a) of this section shall carry out the provisions of that program.

Sec. 108.7  Security program: Form, content, and availability.

# (a)	Each security program required by Sec. 108.5 shall—

(1)	Provide for the safety of persons and property traveling in air transportation and intrastate air transportation against acts of criminal violence and air piracy;

(2)	Be in writing and signed by the certificate holder or any person delegated authority in this matter;

(3)	Include the items listed in paragraph (b) of this section, as required by Sec. 108.5; and

(4)	Be approved by the Administrator.

# (b)	Each security program required by Sec. 108.5 must include the following, as required by that section:

(1)	The procedures and a description of the facilities and equipment used to perform the screening functions specified in Sec. 108.9.

(2)	The procedures and a description of the facilities and equipment used to perform the airplane and facilities control functions specified in Sec.  108.13.

(3)	The procedures used to comply with the applicable requirements of Sec.

108.15 regarding law enforcement officers.

(4)	The procedures used to comply with the requirements of Sec. 108.17 regarding the use of X-ray systems.

(5)	The procedures used to comply with the requirements of Sec. 108.19 regarding bomb and air piracy threats.

(6)	The procedures used to comply with the applicable requirements of Sec.

108.10.

(7)	The curriculum used to accomplish the training required by Sec. 108.23.

(8)	The procedures and a description of the facilities and equipment used to comply with the requirements of Sec. 108.20 regarding explosives detection systems.

# (c)	Each certificate holder having an approved security program shall—

(1)	Maintain at least one complete copy of the approved security program at its principal business office;

(2)	Maintain a complete copy or the pertinent portions of its approved security program or appropriate implementing instructions at each airport where security screening is being conducted;

(3)	Make these documents available for inspection upon request of any Civil Aviation Security Inspector;

(4)	Restrict the distribution, disclosure, and availability of sensitive security information, as defined in part 191 of this chapter, to persons with a need-to-know; and

(5)	Refer requests for sensitive security information by other persons to the Assistant Administrator for Civil Aviation Security.

[Doc. No. 108, 46 FR 3786, Jan. 15, 1981,

as amended by Amdt. 108-3, 50 FR 28893, July 16, 1985;

Amdt. 108-7, 54 FR 36946, Sept. 5, 1989;

Amdt. 108-15, 62 FR 13744, Mar. 21, 1997]

Sec. 108.9   Screening of passengers and property.

(a)	Each certificate holder required to conduct screening under a security program shall use the procedures included, and the facilities and equipment described, in its approved security program to prevent or deter the carriage aboard airplanes of any explosive, incendiary, or a deadly or dangerous weapon on or about each individual’s person or accessible property, and the carriage of any explosive or incendiary in checked baggage.

(b)	Each certificate holder required to conduct screening under a security program shall refuse to transport—

(1)	Any person who does not consent to a search of his or her person in accordance with the screening system prescribed in paragraph (a) of this section; and

(2)	Any property of any person who does not consent to a search or inspection of that property in accordance with the screening system prescribed by paragraph (a) of this section.

(c)	Except as provided by its approved security program, each certificate holder required to conduct screening under a security program shall use the procedures included, and the facilities and equipment described, in its approved security program for detecting explosives, incendiaries, and deadly or dangerous weapons to inspect each person entering a sterile area at each preboarding screening checkpoint in the United States for which it is responsible, and to inspect all accessible property under that person’s control.

(d)	Each certificate holder shall staff its security screening checkpoints with supervisory and non-supervisory personnel in accordance with the standards specified in its security program.

[Doc. No. 108, 46 FR 3786, Jan. 15, 1981,

as amended by Amdt. 108-4, 51 FR 1352, Jan. 10, 1986;

Amdt. 108-5, 52 FR 48509, Dec. 22, 1987;

Doc. No. 26422, Amdt. Nos. 107-6 and 108-10, 56 FR 26522, Aug. 20, 1991]

Sec. 108.10  Prevention and management of hijackings and sabotage attempts.

(a)	Each certificate holder shall—

(1)	Provide and use a Security Coordinator on the ground and in flight for each international and domestic flight, as required by its approved security program; and

(2)	Designate the pilot in command as the inflight Security Coordinator for each flight, as required by its approved security program.

(b)	Ground Security Coordinator. Each ground Security Coordinator shall carry out the ground Security Coordinator duties specified in the certificate holder’s approved security program.

(c)	Inflight Security Coordinator. The pilot in command of each flight shall carry out the inflight Security Coordinator duties specified in the certificate holder’s approved security program.

[Doc. No. 24719, 50 FR 28893, July 16, 1985]

Sec. 108.11   Carriage of weapons.

(a)	No certificate holder required to conduct screening under a security program may permit any person to have, nor may any person have, on or about his or her person or property, a deadly or dangerous weapon, either concealed or unconcealed, accessible to him or her while aboard an airplane for which screening is required unless:

(1)	The person having the weapon is—

(i)	An official or employee of the United States, or a State or political subdivision of a State, or of a municipality who is authorized by his or her agency to have the weapon; or

(ii)	Authorized to have the weapon by the certificate holder and the Administrator and has successfully completed a course of training in the use of firearms acceptable to the Administrator.

(2)	The person having the weapon needs to have the weapon accessible in connection with the performance of his or her duty from the time he or she would otherwise check it in accordance with paragraph (d) of this section until the time it would be returned after deplaning.

(3)	The certificate holder is notified—

(i)	Of the flight on which the armed person intends to have the weapon accessible to him or her at least 1 hour, or in an emergency as soon as practicable, before departure; and

(ii)	When the armed person is other than an employee or official of the United States, that there is a need for the weapon to be accessible to the armed person in connection with the performance of that person’s duty from the time he or she would otherwise check it in accordance with paragraph (d) of this section until the time it would be returned to him or her after deplaning.

(4)	The armed person identifies himself or herself to the certificate holder by presenting credentials that include his or her clear, full-face picture, his or her signature, and the signature of the authorizing official of his or her service or the official seal of his or her service. A badge, shield, or similar may not be used as the sole means of identification.

(5)	The certificate holder—

(i)	Ensures that the armed person is familiar with its procedures for carrying a deadly or dangerous weapon aboard its airplane before the time the person boards the airplane;

(ii)	Ensures that the identity of the armed person is known to each law enforcement officer and each employee of the certificate holder responsible for security during the boarding of the airplane; and

(iii)	Notifies the pilot in command, other appropriate crewmembers, and any other person authorized to have a weapon accessible to him or her aboard the airplane of the location of each authorized armed person aboard the airplane.

(b)	No person may, while on board an airplane operated by a certificate holder for which screening is not conducted, carry on or about that person a deadly or dangerous weapon, either concealed or unconcealed. This paragraph does not apply to—

(1)	Officials or employees of a municipality or a State, or of the United States, who are authorized to carry arms; or

(2)	Crewmembers and other persons authorized by the certificate holder to carry arms.

(c)	No certificate holder may knowingly permit any person to transport, nor may any person transport or tender for transport, any explosive, incendiary or a loaded firearm in checked baggage aboard an airplane. For the purpose of this section, a loaded firearm means a firearm which has a live round of ammunition, cartridge, detonator, or powder in the chamber or in a clip, magazine, or cylinder inserted in it.

(d)	No certificate holder may knowingly permit any person to transport, nor may any person transport or tender for transport, any unloaded firearm in checked baggage aboard an airplane unless—

(1)	The passenger declares to the certificate holder, either orally or in writing before checking the baggage, that any firearm carried in the baggage is unloaded;

(2)	The firearm is carried in a container the certificate holder considers appropriate for air transportation;

(3)	When the firearm is other than a shotgun, rifle, or other firearm normally fired from the shoulder position, the baggage in which it is carried is locked, and only the passenger checking the baggage retains the key or combination; and

(4)	The baggage containing the firearm is carried in an area, other than the flightcrew compartment, that is inaccessible to passengers.

(e)	No certificate holder may serve any alcoholic beverage to a person having a deadly or dangerous weapon accessible to him or her nor may such person drink any alcoholic beverage while aboard an airplane operated by the certificate holder.

(f)	Paragraphs (a), (b), and (d) of this section do not apply to the carriage of firearms aboard air carrier flights conducted for the military forces of the Government of the United States when the total cabin load of the airplane is under exclusive use by those military forces if the following conditions are met:

(1)	No firearm is loaded and all bolts to such firearms are locked in the open position; and

(2)	The certificate holder is notified by the unit commander or officer in charge of the flight before boarding that weapons will be carried aboard the aircraft.

[Doc. No. 108, 46 FR 3786, Jan. 15, 1981,

as amended by Amdt. 108-4, 51 FR 1352, Jan. 10, 1986]

Sec. 108.13  Security of airplanes and facilities.

Each certificate holder required to conduct screening under a security program shall use the procedures included, and the facilities and equipment described, in its approved security program to perform the following control functions with respect to each airplane operation for which screening is required:

(a)	Prohibit unauthorized access to the airplane.

(b)	Ensure that baggage carried in the airplane is checked in by a responsible agent and that identification is obtained from persons, other than known shippers, shipping goods or cargo aboard the airplane.

(c)	Ensure that cargo and checked baggage carried aboard the airplane is handled in a manner that prohibits unauthorized access.

(d)	Conduct a security inspection of the airplane before placing it in service and after it has been left unattended.

Sec. 108.14  Transportation of Federal Air Marshals.

(a)	Each certificate holder shall carry Federal Air Marshals, in the number and manner specified by the Administrator, on each scheduled and public charter passenger operation designated by the Administrator.

(b)	Each Federal Air Marshal shall be carried on a first priority basis and without charge while on official duty, including repositioning flights.

(c)	Each certificate holder shall assign the specific seat requested by a Federal Air Marshal who is on official duty.

[Doc. No. 24714, 50 FR 27925, July 8, 1985]

Sec. 108.15  Law enforcement officers.

(a)	At airports within the United States not governed by Part 107 of this chapter, each certificate holder engaging in scheduled passenger or public charter passenger operations shall—

(1)	If security screening is required for a public charter operation by Sec. 108.5(a), or for a scheduled passenger operation by Sec. 108.5(b) provide for law enforcement officers meeting the qualifications and standards, and in the number and manner specified, in Part 107; and

(2)	When using airplanes with a passenger seating configuration of 31 through 60 seats in a public charter operation for which screening is not required, arrange for law enforcement officers meeting the qualifications and standards specified in Part 107 to be available to respond to an incident, and provide to its employees, including crewmembers, as appropriate, current information with respect to procedures for obtaining law enforcement assistance at that airport.

(b)	At airports governed by Part 107 of this chapter, each certificate holder engaging in scheduled or public charter passenger operations, when using airplanes with a passenger seating configuration of 31 through 60 seats for which screening is not required, shall arrange for law enforcement officers meeting the qualifications and standards specified in Part 107 to be available to respond to an incident and provide its employees, including crewmembers, as appropriate, current information with respect to procedures for obtaining this law enforcement assistance at that airport.

Sec. 108.17  Use of X-ray systems.

(a)	No certificate holder may use an X-ray system within the United States to inspect carry-on or checked articles unless specifically authorized under a security program required by Sec. 108.5 of this part or use such a system contrary to its approved security program. The Administrator authorizes certificate holders to use X-ray systems for inspecting carry-on or checked articles under an approved security program if the certificate holder shows that—

(1)	For a system manufactured before April 25, 1974, it meets either the guidelines issued by the Food and Drug Administration (FDA), Department of Health, Education, and Welfare (HEW) and published in the Federal Register (38 FR 21442, August 8, 1973); or the performance standards for cabinet X-ray systems designed primarily for the inspection of carry-on baggage issued by the FDA and published in 21 CFR 1020.40 (39 FR 12985, April 10, 1974);

(2)	For a system manufactured after April 24, 1974, it meets the standards for cabinet X-ray systems designed primarily for the inspection of carry-on baggage issued by the FDA and published in 21 CFR 1020.40 (39 FR 12985, April 10, 1974);

(3)	A program for initial and recurrent training of operators of the system is established, which includes training in radiation safety, the efficient use of X-ray systems, and the identification of weapons and other dangerous articles;

(4)	Procedures are established to ensure that each operator of the system is provided with an individual personnel dosimeter (such as a film badge or thermoluminescent dosimeter). Each dosimeter used shall be evaluated at the end of each calendar month, and records of operator duty time and the results of dosimeter evaluations shall be maintained by the certificate holder; and

(5)	The system meets the imaging requirements set forth in an approved Air Carrier Security Program using the step wedge specified in American Society for Testing and Materials Standard F792-82.

(b)	No certificate holder may use an X-ray system within the United States unless within the preceding 12 calendar months a radiation survey has been conducted which shows that the system meets the applicable performance standards in 21 CFR 1020.40 or guidelines published by the FDA in the Federal Register of August 8, 1973 (38 FR 21442).

(c)	No certificate holder may use an X-ray system after the system is initially installed or after it has been moved from one location to another, unless a radiation survey is conducted which shows that the system meets the applicable performance standards in 21 CFR 1020.40 or guidelines published by the FDA in the Federal Register of August 8, 1973 (38 FR 21442) except that a radiation survey is not required for an X-ray system that is moved to another location if the certificate holder shows that the system is so designed that it can be moved without altering its performance.

(d)	No certificate holder may use an X-ray system that is not in full compliance with any defect notice or modification order issued for that system by the FDA, unless that Administration has advised the FAA that the defect or failure to comply does not create a significant risk or injury, including genetic injury, to any person.

(e)	No certificate holder may use an X-ray system to inspect carry-on or checked articles unless a sign is posted in a conspicuous place at the screening station and on the X-ray system which notifies passengers that such items are being inspected by an X-ray and advises them to remove all X-ray, scientific, and high-speed film from carry-on and checked articles before inspection. This sign shall also advise passengers that they may request that an inspection be made of their photographic equipment and film packages without exposure to an X-ray system. If the X-ray system exposes any carry-on or checked articles to more than 1 milliroentgen during the inspection, the certificate holder shall post a sign which advises passengers to remove film of all kinds from their articles before inspection. If requested by passengers, their photographic equipment and film packages shall be inspected without exposure to an X-ray system.

(f)	Each certificate holder shall maintain at least one copy of the results of the most recent radiation survey conducted under paragraph (b) or (c) of this section and shall make it available for inspection upon request by the Administrator at each of the following locations:

(1)	The certificate holder’s principal business office; and

(2)	The place where the X-ray system is in operation.

(g)	The American Society for Testing and Materials Standard F792-82, “Design and Use of Ionizing Radiation Equipment for the Detection of Items Prohibited in Controlled Access Areas,” described in this section is incorporated by reference herein and made a part hereof pursuant to 5 U.S.C.  552(a)(1). All persons affected by these amendments may obtain copies of the standard from the American Society for testing and Materials, 1916 Race Street, Philadelphia, PA 19103. In addition, a copy of the standard may be examined at the FAA Rules Docket, Docket No. 24115, 800 Independence Avenue, SW., Washington, DC, weekdays, except Federal holidays, between 8:30 a.m. and 5 p.m.

(h)	Each certificate holder shall comply with X-ray operator duty time limitations specified in its security program.

[Doc. No. 108, 46 FR 3786, Jan. 15, 1981,

as amended by Amdt. 108-1, 50 FR 25656, June 20, 1985;

Doc. No. 26522, Amdt. Nos. 107-6 and 108-10, 56 FR 41425, Aug. 20, 1991;

Doc. No. 26268, Amdt. No. 108-11, 56 FR 48373, Sept. 24, 1991]

Sec. 108.18  Security Directives and Information Circulars.

(a)	Each certificate holder required to have an approved security program for passenger operations shall comply with each Security Directive issued to the certificate holder by the Director of Civil Aviation Security, or by any person to whom the Director has delegated the authority to issue Security Directives, within the time prescribed in the Security Directive for compliance.

(b)	Each certificate holder who receives a Security Directive shall—

(1)	Not later than 24 hours after delivery by the FAA or within the time prescribed in the Security Directive, acknowledge receipt of the Security Directive;

(2)	Not later than 72 hours after delivery by the FAA or within the time prescribed in the Security Directive, specify the method by which the certificate holder has implemented the measures in the Security Directive; and

(3)	Ensure that information regarding the Security Directive and measures implemented in response to the Security Directive are distributed to specified personnel as prescribed in the Security Directive and to other personnel with an operational need to know.

(c)	In the event that the certificate holder is unable to implement the measures contained in the Security Directive, the certificate holder shall submit proposed alternative measures, and the basis for submitting the alternative measures, to the Director of Civil Aviation Security for approval. The certificate holder shall submit proposed alternative measures within the time prescribed in the Security Directive. The certificate holder shall implement any alternative measures approved by the Director of Civil Aviation Security.

(d)	Each certificate holder who receives a Security Directive or Information Circular and each person who receives information from a Security Directive or Information Circular shall—

(1)	Restrict the availability of the Security Directive or Information Circular and information contained in the Security Directive or the Information Circular to those persons with an operational need to know; and

(2)	Refuse to release the Security Directive or Information Circular and information regarding the Security Directive or Information Circular to persons other than those with an operational need to know without the prior written consent of the Director of Civil Aviation Security.

(Approved by the Office of Management and Budget under control number 2120-0098)

[Doc. No. 25953, 54 FR 28984, July 10, 1989]

Sec. 108.19   Security Threats and Procedures.

(a)	Upon receipt of a specific and credible threat to the security of a flight, the certificate holder shall—

(1)	Immediately notify the ground and in-flight security coordinators of the threat, any evaluation thereof, and any countermeasures to be applied; and

(2)	Ensure that the in-flight security coordinator notifies the flight and cabin crewmembers of the threat, any evaluation thereof, and any countermeasures to be applied.

(b)	Upon receipt of a bomb threat against a specific airplane, each certificate holder shall attempt to determine whether or not any explosive or incendiary is aboard the airplane involved by doing the following:

(1)	Conducting a security inspection on the ground before the next flight or, if the airplane is in flight, immediately after its next landing.

(2)	If the airplane is being operated on the ground, advising the pilot in command to immediately submit the airplane for a security inspection.

(3)	If the airplane is in flight, immediately advising the pilot in command of all pertinent information available so that necessary emergency action can be taken.

(c)	Immediately upon receiving information that an act or suspected act of air piracy has been committed, the certificate holder shall notify the Administrator. If the airplane is in airspace under other than United States jurisdiction, the certificate holder shall also notify the appropriate authorities of the State in whose territory the airplane is located and, if the airplane is in flight, the appropriate authorities of the State in whose territory the airplane is to land. Notification of the appropriate air traffic controlling authority is sufficient action to meet this requirement.

[Doc. No. 108, 46 FR 3786, Jan. 15, 1981,

as amended by Amdt. 108-4, 51 FR 1352, Jan. 10, 1986;

Amdt. 108-9, 56 FR 27869, June 17, 1991]

Sec. 108.20  Use of Explosives Detection Systems.

When the Administrator shall require by amendment under Sec. 108.25, each certificate holder required to conduct screening under a security program shall use an explosive detection system that has been approved by the Administrator to screen checked baggage on international flights in accordance with the certificate holder’s security program.

[Doc. No. 25956, 54 FR 36946, Sept. 5, 1989]

Sec. 108.21  Carriage of passengers under the control of armed law enforcement escorts.

(a)	Except as provided in paragraph (e) of this section, no certificate holder required to conduct screening under a security program may carry a passenger in the custody of an armed law enforcement escort aboard an airplane for which screening is required unless—

(1)	The armed law enforcement escort is an official or employee of the United States, of a State or political subdivision of a State, or a municipality who is required by appropriate authority to maintain custody and control over an individual aboard an airplane;

(2)	The certificate holder is notified by the responsible government entity at least 1 hour, or in case of emergency as soon as possible, before departure—

(i)	Of the identity of the passenger to be carried and the flight on which it is proposed to carry the passenger; and

(ii)	Whether or not the passenger is considered to be in a maximum risk category;

(3)	If the passenger is considered to be in a maximum risk category, that the passenger is under the control of at least two armed law enforcement escorts and no other passengers are under the control of those two law enforcement escorts;

(4)	No more than one passenger who the certificate holder has been notified is in a maximum risk category is carried on the airplane;

(5)	If the passenger is not considered to be in a maximum risk category, the passenger is under the control of at least one armed law enforcement escort, and no more than two of these persons are carried under the control of any one law enforcement escort;

(6)	The certificate holder is assured, prior to departure, by each law enforcement escort that—

(i)	The officer is equipped with adequate restraining devices to be used in the event restraint of any passenger under the control of the escort becomes necessary; and

(ii)	Each passenger under the control of the escort has been searched and does not have on or about his or her person or property anything that can be used as a deadly or dangerous weapon;

(7)	Each passenger under the control of a law enforcement escort is—

(i)	Boarded before any other passengers when boarding at the airport where the flight originates and deplaned at the destination after all other deplaning passengers have deplaned;

(ii)	Seated in the rear-most passenger seat when boarding at the airport where the flight originates; and

(iii)	Seated in a seat that is neither located in any lounge area nor located next to or directly across from any exit; and

(8)	A law enforcement escort having control of a passenger is seated between the passenger and any aisle.

(b)	No certificate holder operating an airplane under paragraph (a) of this section may—

(1)	Serve food beverage or provide metal eating utensils to a passenger under the control of a law enforcement escort while aboard the airplane unless authorized to do so by the law enforcement escort.

(2)	Serve a law enforcement escort or the passenger under the control of the escort any alcoholic beverages while aboard the airplane.

(c)	Each law enforcement escort carried under the provisions of paragraph

(a)	of this section shall, at all times, accompany the passenger under the control of the escort and keep the passenger under surveillance while aboard the airplane.

(d)	No law enforcement escort carried under paragraph (b) of this section or any passenger under the control of the escort may drink alcoholic beverages while aboard the airplane.

(e)	This section does not apply to the carriage of passengers under voluntary protective escort.

Sec. 108.23  Training.

(a)	No certificate holder may use any person as a Security Coordinator unless, within the preceding 12 calendar months, that person has satisfactorily completed the security training as specified in the certificate holder’s approved security program.

(b)	No certificate holder may use any person as a crewmember on any domestic or international flight unless within the preceding 12 calendar months or within the time period specified in an Advanced Qualification Program approved under SFAR 58 that person has satisfactorily completed the security training required by Sec. 121.417(b)(3)(v) or Sec. 135.331(b)(3)(v) of this chapter and as specified in the certificate holder’s approved security program. With respect to training conducted under Sec. 121.417 or Sec. 135.331, Whenever a crewmember Who is required to take recurrent training completes the training in the calendar month before or the calendar month after the calendar month in which that training is required, he is considered to have completed the training in the calendar month in which it was required.

[Doc. No. 24719, 50 FR 28893, July 16, 1985,

as amended by Amdt. 108-8, 55 FR 40275, Oct. 2, 1990]

Sec. 108.25  Approval of security programs and amendments.

(a)	Unless otherwise authorized by the Administrator, each certificate holder required to have a security program for a passenger operation shall submit its proposed security program to the Administrator for approval at least 90 days before the date of the intended passenger operations. Within 30 days after receiving the program, the Administrator either approves the program or notifies the certificate holder to modify the program to comply with the applicable requirements of this part. The certificate holder may petition the Administrator to reconsider the notice to modify within 30 days after receiving the notice, and, except in the case of an emergency requiring immediate action in the interest of safety, the filing of the petition stays the notice pending a decision by the Administrator.

(b)	The Administrator may amend an approved security program if it is determined that safety and the public interest require the amendment, as follows:

(1)	The Administrator notifies the certificate holder, in writing, of the proposed amendment, fixing a period of not less than 30 days within which it may submit written information, views, and arguments on the amendment.

(2)	After considering all relevant material, the Administrator notifies the certificate holder of any amendment adopted or rescinds the notice. The amendment becomes effective not less than 30 days after the certificate holder receives the notice, unless the certificate holder petitions the Administrator to reconsider the amendment, in which case the effective date is stayed by the Administrator.

(3)	If the Administrator finds that there is an emergency requiring immediate action with respect to safety in air transportation or in air commerce that makes the procedure in this paragraph impracticable or contrary to the public interest, the Administrator may issue an amendment, effective without stay, on the date the certificate holder receives notice of it. In such a case, the Administrator incorporates the findings, and a brief statement of the reasons for it, in the notice of the amendment to be adopted.

(c)	A certificate holder may submit a request to the Administrator to amend its program. The application must be filed with the Administrator at least 30 days before the date it proposes for the amendment to become effective, unless a shorter period is allowed by the Administrator. Within 15 days after receiving a proposed amendment, the Administrator either approves or denies the request. Within 30 days after receiving from the Administrator a notice of refusal to approve the application for amendment, the applicant may petition the Administrator to reconsider the refusal to amend.

Sec. 108.27  Evidence of compliance.

On request of the Administrator, each certificate holder shall provide evidence of compliance with this part and its approved security program.

[Doc. 24719, 50 FR 28894, July 16, 1985;

50 FR 35535, Aug. 30, 1985; 51 FR 44875, Dec. 12, 1986]

Sec. 108.29   Standards for security oversight.

# (a)	Each certificate holder shall ensure that:

(1)	Each person performing a security-related function for the certificate holder has knowledge of the provisions of this part 108, applicable Security Directives and Information Circulars promulgated pursuant to Sec. 108.18, and the certificate holder’s security program to the extent that the performance of the function imposes a need to know.

(2)	Daily, a Ground Security Coordinator at each airport:

(i)	Reviews all security-related functions for effectiveness and compliance with this part, the certificate holder’s security program, and applicable

Security Directives; and

(ii)	Immediately initiates corrective action for each instance of noncompliance with this part, the certificate holder’s security program, and applicable Security Directives.

# (b)	The requirements prescribed in paragraph (a) of this section apply to all security-related functions performed for the certificate holder whether by a direct employee or a contractor employee.

[Doc. No. 26522, Amdt. Nos. 107-6 and 108-10, 56 FR 41425, Aug. 20, 1991]

Sec. 108.31   Employment standards for screening personnel.

(a)	No certificate holder shall use any person to perform any screening function, unless that person has:

(1)	A high school diploma, a General Equivalency Diploma, or a combination of education and experience which the certificate holder has determined to have equipped the person to perform the duties of the position;

(2)	Basic aptitudes and physical abilities including color perception, visual and aural acuity, physical coordination, and motor skills to the following standards:

(i)	Screeners operating X-ray equipment must be able to distinguish on the X-ray monitor the appropriate imaging standard specified in the certificate holder’s security program. Wherever the X-ray system displays colors, the operator must be able to perceive each color;

(ii)	Screeners operating any screening equipment must be able to distinguish each color displayed on every type of screening equipment and explain what each color signifies;

(iii)	Screeners must be able to hear and respond to the spoken voice and to audible alarms generated by screening equipment in an active checkpoint environment;

(iv)	Screeners performing physical searches or other related operations must be able to efficiently and thoroughly manipulate and handle such baggage, containers, and other objects subjects to security processing; and

(v)	Screeners who perform pat-downs or hand-held metal detector searches of persons must have sufficient dexterity and capability to conduct those procedures on all parts of the persons’ bodies.

(3)	The ability to read, speak, and write English well enough to:

(i)	Carry out written and oral instructions regarding the proper performance of screening duties;

(ii)	Read English language identification media, credentials, airline tickets, and labels on items normally encountered in the screening process;

(iii)	Provide direction to and understand and answer questions from English-speaking persons undergoing screening; and

(iv)	Write incident reports and statements and log entries into security records in the English language.

(4)	Satisfactorily completed all initial, recurrent, and appropriate specialized training required by the certificate holder’s security program.

(b)	Notwithstanding the provisions of paragraph (a)(4) of this section, the certificate holder may use a person during the on-the-job portion of training to perform security functions provided that the person is closely supervised and does not make independent judgments as to whether persons or property may enter a sterile area or aircraft without further inspection.

(c)	No certificate holder shall use a person to perform a screening function after that person has failed an operational test related to that function until that person has successfully completed the remedial training specified in the certificate holder’s security program.

(d)	Each certificate holder shall ensure that a Ground Security Coordinator conducts and documents an annual evaluation of each person assigned screening duties and may continue that person’s employment in a screening capacity only upon the determination by that Ground Security Coordinator that the person:

(1)	Has not suffered a significant dimunition of any physical ability required to perform a screening function since the last evaluation of those abilities;

(2)	Has a satisfactory record of performance and attention to duty; and

(3)	Demonstrates the current knowledge and skills necessary to courteously, vigilantly, and effectively perform screening functions.

(e)	Paragraphs (a) through (d) of this section do not apply to those screening functions conducted outside the United States over which the certificate holder does not have operational control.

(f)	At locations outside the United States where the certificate holder has operational control over a screening function, the certificate holder may use screeners who do not meet the requirements of paragraph (a)(3) of this section, provided that at least one representative of the certificate holder who has the ability to functionally read and speak English is present while the certificate holder’s passengers are undergoing security processing.

[Doc. No. 26522, Amdt. Nos. 107-8 and 108-10, 56 FR 41425, Aug. 20, 1991]

Sec. 108.33  Employment history, verification and criminal history records checks.

(a)	Scope. The following persons are within the scope of this section:

(1)	Each employee or contractor employee covered under a certification made to an airport operator, pursuant to Sec. 107.31(n) of this chapter, made on or after November 23, 1998.

(2)	Each individual issued air carrier identification media that one or more airports accepts as airport approved media for unescorted access within a security identification display area (SIDA) as described in Sec. 107.25 of this chapter.

(3)	Each individual assigned, after November 23, 1998, to perform the following functions:

(i)	Screen passengers or property that will be carried in a cabin of an aircraft of an air carrier required to screen passengers under this part.

(ii)	Serve as an immediate supervisor (checkpoint security supervisor (CSS)), or the next supervisory level (shift or site supervisor), to those individuals described in paragraph (a)(3)(i) of this section.

(b)	Employment history investigations required. Each air carrier must ensure that, for each individual described in paragraph (a) of this section, the following requirements are met:

(1)	The individual has satisfactorily undergone Part 1 of an employment history investigation. Part 1 consists of a review of the previous 10 years of employment history and verifications of the 5 employment years preceding the date the employment history investigation is initiated as provided in paragraph (c) of this section; and

(2)	If required by paragraph (c)(5) of this section, the individual has satisfied Part 2 of the employment history investigation. Part 2 is the process to determine if the individual has a criminal record. To satisfy Part 2 of the investigation the criminal records check must not disclose that the individual has been convicted or found not guilty by reason of insanity, in any jurisdiction, during the 10 years ending on the date of such investigation, of any of the crimes listed below:

(i)	Forgery of certificates, false marking of aircraft, and other aircraft registration violation, 49 U.S.C. 46306;

(ii) 	Interference with air navigation, 49 U.S.C. 46308;

(iii)	Improper transportation of a hazardous material, 49 U.S.C. 46312;

(iv)	Aircraft piracy, 49 U.S.C. 46502;

(v)	Interference with flightcrew members or flight attendants, 49 U.S.C. 46504;

(vi)	Commission of certain crimes aboard aircraft in flight, 49 U.S.C. 46506;

(vii)	Carrying a weapon or explosive aboard aircraft, 49 U.S.C. 46505;

(viii)	Conveying false information and threats, 49 U.S.C. 46507;

(ix)	Aircraft piracy outside the special aircraft jurisdiction of the United States, 49 U.S.C. 46502(b);

(x)	Lighting violations involving transporting controlled substances, 49U.S.C. 46315;

(xi)	Unlawful entry into an aircraft or airport area that serves air carriers or foreign air carriers contrary to established security requirements, 49 U.S.C. 46314;

(xii)	Destruction of an aircraft or aircraft facility, 18 U.S.C. 32;

(xiii)	Murder;

(xiv)	Assault with intent to murder;

(xv)	Espionage;

(xvi)	Sedition;

(xvii)	Kidnapping or hostage taking;

(xviii)	Treason;

(xix)	Rape or aggravated sexual abuse;

(xx)	Unlawful possession, use, sale, distribution, or manufacture of an explosive or weapon;

(xxi)	Extortion;

(xxii)	Armed robbery;

(xxiii)	Distribution of, or intent to distribute, a controlled substance;

(xxiv)	Felony arson; or

(xxv)	Conspiracy or attempt to commit any of the aforementioned criminal acts.

(c)	Investigative steps. Part 1 of the employment history investigations must be completed on all persons described in paragraph (a) of this section.  If required by paragraph (c)(5) of this section, Part 2 of the employment history investigation must also be completed on all persons listed in paragraph (a) of this section.

(1)	The individual must provide the following information on an application:

(i)	The individual’s full name, including any aliases or nicknames;

(ii)	The dates, names, phone numbers, and addresses of previous employers, with explanations for any gaps in employment of more than 12 consecutive months, during the previous 10-year period;

(iii)	Any convictions during the previous 10-year period of the crimes listed in paragraph (b)(2) of this section.

(2)	The air carrier must include on the application form a notification that the individual will be subject to an employment history verification and possibly a criminal records check.

(3)	The air carrier must verify the identity of the individual through the presentation of two forms of identification, one of which must bear the individual’s photograph.

(4)	The air carrier must verify the information on the most recent 5 years of employment history required under paragraph (c)(1)(ii) of this section.  Information must be verified in writing, by documentation, by telephone, or in person.

(5)	If one or more of the conditions (triggers) listed in Sec. 108.33(c)(5) (i) through (iv) exist, the employment history investigation must not be considered complete unless Part 2 is accomplished. Only the air carrier may initiate Part 2. Part 2 consists of a comparison of the individual’s fingerprints against the fingerprint files of known criminals maintained by the Federal Bureau of Investigation (FBI). The comparison of the individual’s fingerprints must be processed through the FAA. The air carrier may request a check of the individual’s fingerprint-based criminal record only if one or more of the following conditions exist:

(i)	The individual does not satisfactorily account for a period of unemployment of 12 consecutive months or more during the previous 10-year period.

(ii)	The individual is unable to support statements made on the application form.

(iii)	There are significant inconsistencies in the information provided on the application.

(iv)	Information becomes available to the air carrier during the investigation indicating a possible conviction for one of the crimes listed in paragraph (b)(2) of this section.

(d)	Individual notification. Prior to commencing the criminal records check, the air carrier must notify the affected individuals and identify a point of contact for follow-up. An individual who chooses not to submit fingerprints may not be granted unescorted access privilege and may not be allowed to hold screener or screener supervisory positions.

(e)	Fingerprint processing. If a fingerprint comparison is necessary under paragraph (c)(5) of this section to complete the employment history investigation the air carrier must collect and process fingerprints in the following manner:

(1)	One set of legible and classifiable fingerprints must be recorded on fingerprint cards approved by the FBI and distributed by the FAA for this purpose.

(2)	The fingerprints must be obtained from the individual under direct observation by the air carrier or a law enforcement officer. Individuals submitting their fingerprints must not take possession of their fingerprint card after they have been fingerprinted.

(3)	The identify of the individual must be verified at the time fingerprints are obtained. The individual must present two forms of identification, one of which must bear the individual’s photograph.

(4)	The fingerprint card must be forwarded to FAA at the location specified by the Administrator.

(5)	Fees for the processing of the criminal records checks are due upon application. Air carriers must submit payment through corporate check, cashier’s check, or money order made payable to “U.S. FAA,” at the designated rate for each fingerprint card. Combined payment for multiple applications is acceptable. The designated rate for processing the fingerprint cards is available from the local FAA security office.

(f)	Determination of arrest status. In conducting the criminal record checks required by this section, the air carrier must not consider the employment history investigation complete unless it investigates arrest information for the crimes listed in paragraph (b)(2) of this section for which no disposition has been recorded and makes a determination that the arrest did not result in a disqualifying conviction.

(g)	Availability and correction of FBI records and notification of disqualification.

(1) At the time Part 2 is initiated and the fingerprints are collected, the air carrier must notify the individual that a copy of the criminal record received from the FBI will be made available to the individual if requested in writing. When requested in writing, the air carrier must make available to the individual a copy of any criminal record received from the FBI.

(2)	Prior to making a final decision to deny authorization to an individual described in paragraph (a) of this section, the air carrier must advise the individual that the FBI criminal record discloses information that would disqualify him/her from positions covered under this rule and provide him/her with a copy of their FBI record if requested.

(3)	The air carrier must notify an individual that a final decision has been made to forward or not forward a letter of certification for unescorted access to the airport operator, or to grant or deny the individual authority to perform screening functions listed under paragraph (a)(3) of this section.

(h)	Corrective action by the individual. The individual may contact the local jurisdiction responsible for the information and the FBI to complete or correct the information contained in his/her record before the air carrier makes any decision to withhold his/her name from a certification, or not grant authorization to perform screening functions subject to the following conditions:

(1)	Within 30 days after being advised that the criminal record received from the FBI discloses disqualifying information, the individual must notify the air carrier, in writing, of his/her intent to correct any information believed to be inaccurate.

(2)	Upon notification by an individual that the record has been corrected, the air carrier must obtain a copy of the revised FBI record prior to making a final determination.

(3)	If no notification is received within 30 days, the air carrier may make a final determination.

(i)	Limits on dissemination of results. Criminal record information provided by the FBI must be used solely for the purposes of this section, and no person may disseminate the results of a criminal record check to anyone other than:

(1)	The individual to whom the record pertains or that individual’s authorized representative;

(2)	Air carrier officials with a need to know; and

(3)	Others designated by the Administrator.

(j)	Employment status while awaiting criminal record checks. Individuals who have submitted their fingerprints and are awaiting FBI results may perform work details under the following conditions:

(1)	Those seeking unescorted access to the SIDA must be escorted by someone who has unescorted SIDA access privileges;

(2)	Those applicants seeking positions covered under paragraphs (a)(3) and (a)(4) of this section, may not exercise any independent judgments regarding those functions.

(k)	Recordkeeping.

(1)	The air carrier must physically maintain and control

Part 1 employment history investigation file until 180 days after the termination of the individual’s authority for unescorted access or termination from positions covered under paragraph (a)(3) of this section.  Part 1 of the employment history investigation, completed on screening personnel must be maintained at the airport where they perform screening functions. Part 1 of the employment history investigation file must consist of the following:

(i)	The application;

(ii)	The employment verification information obtained by the employer;

(iii)	the names of those from whom the employment verification information was obtained;

(iv)	The date and the method of how the contact was made; and

(v)	Any other information as required by the Administrator.

(2)	The air carrier must physically maintain, control and when appropriate destroy Part 2, the criminal record file, for each individual for whom a fingerprint comparison has been made. Part 2 must be maintained for 180 days after the termination of the individual’s authority for unescorted access or after the individual ceases to perform screening functions. Only direct air carrier employees may carry out Part 2 responsibilities. Part 2 must consist of the following:

(i)	The results of the record check; or

(ii)	Certification from the air carrier that the check was completed and did not uncover a disqualifying conviction.

(3)	The files required by this paragraph must be maintained in a manner that is acceptable to the Administrator and in a manner that protects the confidentiality of the individual.

(l)   Continuing responsibilities.

(1)	Any individual authorized to have

unescorted access privilege to the SIDA or who performs functions covered under paragraph (a)(3) of this section, who is subsequently convicted of any of the crimes listed in paragraph (b)(2) of this section must, within 24 hours, report the conviction to the air carrier and surrender the SIDA access medium or any employment related identification medium to the issuer.

(2)	If information becomes available to the air carrier indicating that an individual has a possible conviction for one of the disqualifying crimes in paragraph (b)(2) of this section, the air carrier must determine the status of the conviction and, if the conviction is confirmed:

# (i)	Immediately revoke access authorization for unescorted access to the SIDA; or

# (ii)	Immediately remove the individual from screening functions covered under paragraph (a)(3) of this section.

(m)	Air carrier responsibility. The air carrier must:

(1)	Designate an individual(s), in the security program, to be responsible for maintaining and controlling the employment history investigation for those whom the air carrier has made a certification to an airport operator under Sec. 107.31(n)(1) of this chapter and for destroying the criminal record files when their maintenance is no longer required by paragraph (k)(2) of this section.

(2)	Designate individual(s), in the security program, to maintain and control Part 1 of the employment history investigations of screeners whose files must be maintained at the location or station where the screener is performing his or her duties.

(3)	Designate individual(s), in the security program, to serve as the contact to receive notification from an individual applying for either unescorted access or those seeking to perform screening functions of his or her intent to seek correction of his or her criminal record with the FBI.

(4)	Designate an individual(s), in the security program, to maintain and control Part 2 of the employment history investigation file for all employees, contractors, or others who undergo a fingerprint comparison at the request of the air carrier.

(5)	Audit the employment history investigations performed in accordance with this section. The audit process must be set forth in the air carrier approved security program.

[Amdt. 108-17, 63 FR 51220, Sept. 24, 1998; 63 FR 57161, Oct. 26, 1998]