DEPARTMENT OF TRANSPORTATION

FEDERAL AVIATION ADMINISTRATION

A18SW

Revision 2

Fairchild Aircraft, Inc.

SA227-CC

SA227-DC (C-26B)

November 14, 1996

TYPE CERTIFICATE DATA SHEET A18SW

Type Certificate Holder:	Fairchild Aircraft, Inc.

San Antonio, Texas  78279-0490

I - Model SA227-CC, 21 PCLM, Commuter Category, FAR 23, Approved June 25, 1990.  * NOTE 10

Engines	Two Garrett (Airesearch) TPE331-11U-612G

Fuel	Aviation turbine fuels	Garrett Specification

Type A	EMS53111

Type A-1	EMS53112

Class A-JP4 and	EMS53113

Class B-Type B

Type JP-5	EMS53116

Type JP-8	EMS53112

(Fuel shall conform to the specification as listed or to subsequent revisions thereof).

(See Note 3)

Oil	MIL-L-23699B conforming to Garrett Engine Division Specification EMS53110 Type

II.

Engine Limits	Static Sea Level Ratings.

|  | Shaft | Gas | Prop | Exhaust Gas |
| --- | --- | --- | --- | --- |
|  | Horse | Gen. | Shaft | Temp. (EGT) |
|  | Power | Speed | Speed | (Single Red Line) |
|  | (S.H.P.) | (R.P.M) | (R.P.M.) | (°C) |
| Take-off (5-min) | 1,000 | 41730* | 1591* | 650 |
| Dry |  |  |  |  |
| Take-off (5-min) | 1,100 | 41730* | 1591* | 650 |
| Wet |  |  |  |  |
| Max Continous-Dry | 1,000 | 41730* | 1591* | 650 |
| Starting Limit |  |  |  |  |
| (1-sec) | - | - | - | 770 |

*(See Note 4)

Oil Temps	Minus 40°C to 110°C (normal operations)

Minus 40°C to 127°C (ground operations only)

I - Model SA227-CC, 21 PCLM, Commuter Category, FAR 23, Approved June 25, 1990.  * NOTE 10 (Cont'd)

Propeller and	Number	2

Propeller limits	Make	McCauley

Model	4HFR34C652()/()-L106LA-0

Diameter	106 inches

Pitch At	30 in. station

McCauley Propeller

Assembly Number

D-5928	D-6933

Feathered	88.9° ± 0.5°	88.5° ± 0.5°

Flight Idle	15.0° ± 0.2°	15.0° ± 0.2°

Start Locks	9.0° ± 0.5°	6.0° ± 0.5°

Full Reverse	-5.0° ± 0.5°	-5.0° ± 0.5°

Airspeed Limits			Altitude	Speed

(ft.)	(Knots CAS)

Maximum	17,800	248

Operating	18,000	247

Speed	20,000	237

23,000	223

25,000	214

Maneuvering

@ 16,000#	all	183

Flaps Full Ext.		166

1/2 Ext.		180

1/4 Ext.		215

Ldg Gear Ext.		176

Ldg Gear Oper.		176

C.G. Range	262.8 (16.41% MAC) to 277.0 (36.0% MAC) at 16,500 lbs.

(Inches Aft of	257.0 (8.40% MAC) to 277.0 (36.0% MAC) at 11,000 lbs. and below.

Datum)	Straight line variation between points given.

Note:  Gear Retraction will not move the c.g. beyond approved limits if the airplane is

loaded within the gear down envelope.

Empty Weight	None

C.G. Range

Maximum Weight (lbs.)	Ramp	16,600

(See Note 6)	Take-off	16,500

Landing	15,675

Max. Zero Fuel	14,500

Maximum Operating	25,000 feet.

Altitude

Minimum Crew	One pilot except as otherwise required by the Airplane Flight Manual (See Note 9).

No. Seats	Maximum 21 (crew at + 111.0).  (Maximum of 19 passengers).  See AFM for loading

instructions for crew and passenger loading.

Maximum Baggage	Rear Compartment:  850 lbs. (+473.4)

and/or Equipment	Nose Compartment:  800 lbs. (+46.7)

Local loading on cargo and passenger compartment floor:  150 lbs./sq. ft.

I - Model SA227-CC, 21 PCLM, Commuter Category, FAR 23, Approved June 25, 1990.  * NOTE 10 (Cont'd)

Fuel Capacity	652 gal. total (324 gal. usable in each of 2 wing tanks).

See Note 1 for data on unusable fuel.

Oil Capacity	14.1 qt. total (3.8 qt. usable in each engine oil tank).

See Note 1 for data on unusable oil.

Control Surface		Wings Flaps			36° ± 1°	down

Main Surface

Aileron	18.5° ± 1°	up	21.5 ± 1°	down

Elevator	30° ± 1°	up	15° ± 1°	down

Rudder	25° ± 1°	right	25° ± 1°	left

Stabilizer (mechanical stops):

2.40° ±.20° L.E. up		7.80° ± 0.20° L.E. down

(electrical stops):

0.2° ± .05° before mechanical stops

Tabs (Main surface in Neutral)

Aileron	20° + 2°, -1°	up	20° + 2°, -1°	down

Rudder	25° ± 1.5°	right	25° ± 1.5°	left

Serial Nos.	CC-790 and up.  (See Note 8)

Datum	Located 274.1 inches forward of wing main (forward) spar centerline.

Leveling Means	Lateral:	Nose baggage compartment door sill.

Longitudinal:	Nose baggage compartment floor.

Certification Basis	FAR Part 23 through Amendment 23-34 plus Amendment 23-39; equivalent safety

finding per FAA letter dated September 20, 1990; FAR Part 36, SFAR 27 through

Amendment 5 (See Note 6).  Approved for flight into known icing in accordance with

FAR 23.1419.

Production Basis:	Production Certificate No. 6SW.

Equipment	The basic required equipment, as prescribed in the applicable airworthiness regulations

(See Certification Basis) must be installed in the aircraft for certification.  Fairchild

Drawing No. 27-10044 "Equipment List, Model SA227-CC" listing of all additional

required equipment as well as optional installations approved by the FAA.  (See Note 9)

II.  Model SA227-DC (C-26B), 21 PCLM, Commuter Category, FAR 23, Approved September 29, 1990 (See Note 7 and 11).

Engines	Two Garrett (Airesearch) TPE331-12UA-701G or TPE331-12UAR-701G or

TPE331-12UHR-701G.

Fuel	Aviation turbine fuels	Garrett Specification

Type A	EMS53111

Type A-1	EMS53112

Class A-JP4 and	EMS53113

Class B-Type B

Type JP-5	EMS53116

Type JP-8	EMS53112

(Fuel shall conform to the specification as listed or to subsequent revisions thereof).

(See Note 3).

Oil	MIL-L-23699B conforming to Garrett Engine Division Specification EMS531100

Type II.

II.  Model SA227-DC (C-26B), 21 PCLM, Commuter Category, FAR 23, Approved September 29, 1990 (See Note 7 and 11).(Cont'd)

Engine Limits	Static Sea Level Ratings.

|  | Shaft | Gas | Prop | Exhaust Gas |
| --- | --- | --- | --- | --- |
|  | Horse | Gen. | Shaft | Temp. (EGT) |
|  | Power | Speed | Speed | (Single Red Line) |
|  | (S.H.P.) | (R.P.M.) | (R.P.M.) | (°C) |
| Take-off (5-min) | 1,100 | 41730* | 1591* | 650 |
| Dry |  |  |  |  |
| Take-off (5-min) | 1,100 | 41730* | 1591* | 650 |
| Wet |  |  |  |  |
| Max Continous-Dry | 1,000 | 41730* | 1591* | 650 |
| Starting Limit |  |  |  |  |
| (1-sec) | - | - | - | 770 |

*(See Note 4)

Oil Temps	Minus 40°C to 110°C (normal operations)

Minus 40°C to 127°C (ground operations only)

Propeller and	Number	2	2

Propeller limits	Make	McCauley	McCauley

Model	4HFR34C663()/()-L106LA-0	4HFR34C652()/()-L106KA-0

Diameter	106 inches	106 inches

Pitch At	30 in. station	30 in. station

McCauley Propeller

Assembly Number

D-5928	D-6933	D-7274

Feathered	88.9° ± 0.5°	8.5° ± 0.5°	88° ± 0.2°

Flight Idle	15.0° ± 0.2°	5.0° ± 0.2°	16.0° ± 1.0°

Start Locks	9.0° ± 0.5°	6.0° ± 0.5°	6.0° ± 0.2°

Full Reverse	-5.0° ± 0.5°	5.0° ± 0.5°	-4.0° ± 0.2°

Airspeed Limits		Altitude	Speed

(ft.)	(Knots CAS)

Maximum	17,800	248

Operating	18,000	247

Speed	20,000	237

23,000	223

25,000	214

Maneuvering

@ 16,000#	all	183

Flaps Full Ext.		166

1/2 Ext.		180

1/4 Ext.		215

Ldg Gear Ext.		176

Ldg Gear Oper.		176

C.G. Range	262.8 (16.41% MAC) to 277.0 (36.0% MAC) at 16,500

Gear Down	257.0 (8.40% MAC) to 277.0 (36.0% MAC) at 11,000 lbs. and below.

(Inches Aft of	Straight line variation between points given.

Datum)

Note:  Gear Retraction will not move the c.g. beyond approved limits if the airplane is

loaded within the gear down envelope.

II.  Model SA227-DC (C-26B), 21 PCLM, Commuter Category, FAR 23, Approved September 29, 1990 (See Note 7 and 11).(Cont'd)

Empty Weight	None

C.G. Range

Maximum Weight	Ramp	16,600

(lbs.)	Take-off	16,500

(See Note 6)	Landing	15,675

Max. Zero Fuel	14,500

Maximum Oper.	25,000 feet.

Altitude

Minimum Crew	One pilot except as otherwise required by the Airplane Flight Manual (See Note 9.)

No. Seats	Maximum 21 (crew at + 111.0).  (Maximum of 19 passengers).

See AFM for loading instructions for crew and passenger loading.

Maximum Baggage	Rear Compartment:  850 lbs.  (+473.4)

and/or Equipment	Nose Compartment:  800 lbs.  (+46.7)

Local loading on cargo and passenger compartment floor:  150 lbs./sq. ft.

Fuel Capacity	652 gal. total (324 gal. usable in each of 2 wing tanks.  See Note 1 for data on unusable

fuel.

Oil Capacity	14.1 qt. total (3.8 qt. usable in each engine oil tank).

See Note 1 for data on unusable oil.

Control Surface		Wings Flaps			36° ± 1°	down

Main Surface

Aileron	18.5° ± 1°	up	21.5 ± 1°	down

Elevator	30° ± 1°	up	15° ± 1°	down

Rudder	25° ± 1°	right	25° ± 1°	left

Stabilizer (mechanical stops):

2.40° ± .20° L.E. up	7.80° ± .20° L.E. down

(electrical stops):

0.2° ± .05° before mechanical stops

Tabs (Main surface in Neutral)

Aileron	20° + 2°, -1°	up;	20° + 2°,-1°	down

Rudder	25° ± 1.5°	right;	25° ± 1.5°	left

Serial Nos.	DC-784 and up.  (See Notes 7 and 8.)

Datum	Located 274.1 inches forward of wing main (forward) spar centerline.

Leveling Means	Lateral:	Nose baggage compartment door sill.

Longitudinal:	Nose baggage compartment floor.

Certification Basis:	FAR Part 23 through Amendment 23-34 plus Amendment 23-39; equivalent safety

finding per FAA letter dated September 20, 1990; FAR Part 36, SFAR 27 through

Amendment 5 (See Note 6).  Approved for flight into known icing in accordance with

FAR 23.1419.

Production Basis:	Production Certificate No. 6SW.

II.  Model SA227-DC (C-26B), 21 PCLM, Commuter Category, FAR 23, Approved September 29, 1990 (See Note 7 and 11).(Cont'd)

Equipment	The basic required equipment, as prescribed in the applicable airworthiness regulations

(See Certification Basis) must be installed in the aircraft for certification.  Fairchild

Drawing No. 27-10045 "Equipment List, Model SA227-DC" contains listing of all

additional required equipment as well as optional installations approved by the FAA.

(See Note 9)

Note 1.	Current weight and balance report, together with list of equipment included in certificated empty weight, and

loading instructions when necessary, must be provided for each aircraft at the time of original certification.

The airplane must be loaded so that the C.G. is within the specified limits at all times.  Empty weight and

corresponding center of gravity location must include:

Unusable fuel	27 lbs.  (+282)

Unusable oil	12 lbs.  (+205)

Unusable AWI	16 lbs.  (+298)

Note 2.	All placards required in the approved AFM must be installed in the appropriate locations.

Note 3.	Emergency use of MIL-G-5572D, 80/87, aviation gasoline permitted not to exceed 1,000 gallons per engine

for each 100 hours of engine operation.  Emergency use of MIL-G-5572D, Grade 100/130 (low lead),

aviation gasoline permitted not exceed 250 gallons per engine for each 100 hours of engine operation with

the total use limited to 7,000 gallons during any 3,000-hour period.  Jet fuel and aviation gasoline may be

mixed in any proportion.  If 25% or more aviation gasoline is used, add 1 quart of MIL-L-6082 specification

grade 1065 or 1100 piston engine oil per 100 gallons of aviation gasoline to provide fuel pump lubrication.

Note:  The amount of aviation gasoline used must be recorded in the Engine Log Book.  Fuel System

Icing Inhibitor MIL-T-27686E fuel additive approved not to exceed 0.15 percent by volume.  No fuel

system anti-icing credit is allowed.

Note 4.	The maximum propeller shaft overspeed limit is 1686 RPM (106%) for 5 seconds and 1615 RPM (101.5%

for 5 minutes).  100% is defined as 1591 RPM.

Note 5.	The Airworthiness Limitations Manual ST-UN-M003 contains overhaul times, replacements times and

special inspections required for continued airworthiness.

Note 6.	Compliance with SFAR 27-5.  "Fuel Venting and Exhaust Emissions Requirements for Turbine Engine

Powered Airplanes" is equivalent to compliance with FAR Part 34, effective September 10, 1990.

Note 7.	The C-26B is an SA227-DC airplane manufactured in accordance with Fairchild drawing 27-10048.  These

airplanes are identified by the letter "M" at the end of the serial number.

Note 8.	The manufacturer has elected to end the serial numbers of airplanes not affected by Note 7 with the letter

"B".

Note 9.	Approval for single-pilot operation is based on the instrument/avionics arrangement shown by Fairchild

Drawing 27-86081 or Drawing 27-88025 (C-26B).  Any significant deviation from that arrangement must be

evaluated for single pilot suitability.

Note 10.	The SA227-CC airplane may be converted to a Model SA227-DC in accordance with FAI drawing 27-14167

initial release.

Note 11.	The SA227-DC airplane may be converted to a Model SA227-CC in accordance with FAI drawing 27-14140

initial release plus EOS A-1 and A-2.

...END...